- Computer Music Definitions

| ### monotimbral | Capable of only having one tone(instrument) active at a time. Contrast with polytimbral. |
| --- | --- |
| ### polytimbral | Capable of producing the tones of more than one instrument simultaneously. Contrast with monotimbral. |
| ### polyphonic | Able to play more than one note at a time. The opposite of monophonic. |
| ### MIDI | Short for “Musical Instrument Digital Interface.” Midi is an industry standard for communication between digital musical instruments and software. It does not transmit actual audio signals or data, but is more like a way to transmit the information that would be contained in a musical score. Midi data consists of “events” such as “note on” and “note off” as well as Midi controller signals for changing parameters like volume and stereo position, as well as events for selecting the currently active “program” or instrument that is playing on a device. |
| ### MIDI controller | One of 128 predefined control parameters that are part of the Midi specification. Each can take on a value between 0 and 127 and several have predefined functions. For example, Midi controller #7 is the volume level for the current channel and Midi controller #64 is usually reserved for transmitting the state of a piano-like sustain pedal. Which Midi controllers a particular instrument responds to as well as how it responds is defined by the instrument’s manufacturer or programmer. |
| ### MIDI program change | A “program” on a digital musical instrument is a collection of settings that determine what the instrument will sound like when it is played (also called a “patch,” “sound,” “instrument,” or occasionally “preset”). Most of these instruments are capable of switching between multiple programs residing in the machine’s memory. A MIDI Program Change is a Midi message that tells an instrument to change from its current program to one specified by the message. |
| ### OMS | The “Open Music System” from Opcode, Inc. OMS is a software architecture for allowing music software running on the same computer to communicate via Midi events. It also functions as a driver for external Midi hardware devices and provides an interface for transparently connecting hardware and software according to the user’s needs. |
| ### oscillator | A hardware or software module which outputs a periodic signal (a “waveform”). See also sine wave, square wave, pulse wave, saw wave, etc. |
| ### operator | An abstraction of some part of a software algorithm for synthesizing sound. Frequently, it is an oscillator that is part of a configuration where two or more oscillators are modulating each other’s signals. See also frequency modulation, amplitude modulation, carrier, and modulator. |
| ### frequency modulation | A process by which one signal is made to vary (“modulate”) the frequency of another signal over time. In audio synthesis, FM is usually between two oscillators producing simple waveforms (sometimes called operators). The signal being modulated is called the carrier and the other one the modulator. |
| ### amplitude modulation | A process by which one signal is made to vary (“modulate”) the amplitude of another signal over time. In audio synthesis, AM is often between two oscillators producing simple waveforms (sometimes called operators). The signal being modulated is called the carrier and the other one the modulator. The result of amplitude modulation is similar to ring modulation but includes all of the frequencies of the carrier signal as well. |
| ### ring modulation | A process by which two audio signals are multiplied together. The resulting output signal is a combination of all the sums and differences between frequencies in the first signal and frequencies in the second signal. |
| ### AND | A logical operation comparing two boolean or truth values (true or false) which only returns true if both inputs are true. In computers, all information is represented as strings of zeros and ones (binary numbers) which can treated as false and true values respectively. When performed on binary numbers, the AND operation operates on each digit of the numbers independently. |
| ### OR | A logical operation comparing two boolean or truth values (true or false) which only returns false if both inputs are false. In computers, all information is represented as strings of zeros and ones (binary numbers) which can treated as false and true values respectively. When performed on binary numbers, the OR operation operates on each digit of the numbers independently. |
| ### Exclusive OR | Also called XOR. A logical operation comparing two boolean or truth values (true or false) which only returns true when its inputs are different. In computers, all information is represented as strings of zeros and ones (binary numbers) which can treated as false and true values respectively. When performed on binary numbers, the XOR operation operates on each digit of the numbers independently. |
| ### sine wave | A periodic signal consisting of a single “pure” frequency. |
| ### square wave | A periodic signal which spends half of the time “high” and the other half “low.” It is a harmonic tone consisting of a fundamental frequency and all odd multiples of the fundamental frequency with relative strengths of one over the multiple. See also pulse wave. |
| ### saw wave | A periodic signal which repeatedly “ramps up” linearly from a minimum value to a maximum value. It is a harmonic tone consisting of a fundamental frequency and all multiples of the fundamental frequency with relative strengths of one over the multiple. |
| ### pulse wave | A periodic rectangular signal which spends X percent of its time “high” and the other 100-X percent “low.” The percentage of time spent high is called the “duty cycle.” A pulse wave with a duty cycle of 50% is identical to a square wave. |
| ### pulse width modulation | A method of synthesizing dynamic tones by varying over time the duty cycle of a pulse wave oscillator. See also pulse wave. |
| ### audio rate | The sampling rate of audio signals in a synthesizer. The audio rate affects the quality of the signals as well as the maximum frequency which may be produced. |
| ### control rate | The sampling rate of control (modulation) signals in a synthesizer. Usually, the control rate is significantly slower than the audio rate since a faster rate of change would not be audible or is not needed. If the control rate is too slow, however, sounds may not transition smoothly between values of time-varying parameters. |
| ### sampling rate | Digital signals are sampled – made up of a stream of individual values – instead of being continuous like analog signals. The sampling rate defines how many discreet values of a signal are contained within one second. |
| ### filter | A filter removes or attenuates a specified range of frequencies that make up a signal. Digital filters are implemented by summing time-delayed copies of a signal with the original. |
| ### finite-impulse response filter | Also FIR filter. A type of digital filter whose output always terminates regardless of its input. It is implemented by multiplying an array of values called the “impulse response” by the array of the last n samples of the input signal (where n is the length of the impulse response) and summing the resulting products together. |
| ### LFO | A “low-frequency oscillator.” An oscillator that usually has a frequency below the audible range for humans (less than 20Hz). They are primarily used as modulation sources to vary the parameters of sound synthesis in a cyclical fashion. The oscillators often produce simple periodic waveforms such as sine waves, square waves, or saw waves. |
| ### modulation source | Anything that modifies over time some parameter of a sound which is being generated by a synthesizer. Examples of common sources are LFOs, envelopes, and Midi controllers. |
| ### envelope | A modulation source which usually varies over the entire length of a note in a non-periodic fashion. Often, an envelope is made up of multiple, adjoining, linear segments called stages. The length (in seconds) of each stage and the value towards which it is moving are configurable. Envelopes often start at a value of zero, become positive and then return to zero at their end. This makes them very useful for controlling the amplitude of a note. |
| ### key follow | A modulation source that maps a value representing the current key (the Midi note number) onto a scale of values that is useful as an input to some synthesis parameter. For example, a key follow is used to allow the pitch of an oscillator to correctly be set based on the Midi Note On message received or to allow the cutoff frequency of a filter to change across the keyboard in a natural way. |
| ### waveshaper |  |
| ### synthesizer |  |
| ### sequencer |  |
| ### GUI |  |
| ### full-scale |  |
| ### carrier |  |
| ### modulator |  |
| ### aliasing |  |